# LiSAT: Data-Driven Semantic-Adaptive Waveform Optimization for 6G Internet of Everything Networks

# Evelio Astaiza Hoyos1, Héctor Fabio Bermúdez-Orozco1, * and Juan Esteban Astaiza Fuenmayor 2

1	Electronic Engineering Programme, Faculty of Engineering, University of Quindío, Armenia 630004, Quindío, Colombia; eastaiza@uniquindio.edu.co (E.A.H.); hfbermudez@uniquindio.edu.co (H.F.B.-O)

2	Systems and Computation Engineering Programme, Faculty of Engineering, University of Quindío, Armenia 630004, Quindío, Colombia; juanastaizafuenmayor@gmail.com (J.E.A.F)

*	Correspondence: hfbermudez@uniquindio.edu.co; Tel.: +057 3206671107

Abstract—The proliferation of heterogeneous devices in sixth-generation (6G) Internet of Everything (IoE) networks requires intelligent physical-layer solutions capable of jointly optimizing semantic fidelity, energy efficiency, and end-to-end latency under highly dynamic doubly-dispersive channel conditions. This paper proposes LiSAT (Lightweight Semantic-Adaptive Transceiver), an AI-native transceiver framework that integrates three tightly coupled modules: (i) a Lightweight Semantic Codec (CSL) based on a four-layer INT8-quantized convolutional neural network (~200 KB), (ii) a Multi-Waveform Estimation and Reconstruction Delay-Doppler (MERD) module that employs a permanent AFDM pilot layer to estimate channel delay-Doppler profiles with 8–12% overhead, and (iii) an Asynchronous Advantage Actor-Critic (A3C) deep reinforcement learning agent (~50 KB INT8) executed on an ARM Cortex-M55 processor to dynamically select among OTFS, OCDM, and AFDM waveforms. The proposed framework is evaluated in four representative 6G communication scenarios, namely high-mobility V2X, THz industrial communications, massive IoE deployments, and non-stationary UAV networks. Simulation results show that LiSAT achieves a 23% symbol error rate reduction compared with OFDM in V2X environments, a 22.5% improvement in semantic efficiency for THz links, and a 41.9% increase in global utility (U = 0.742 versus U = 0.523), while operating within a 256 KB SRAM budget. Furthermore, convergence to a local stationary ε-optimal policy is established under standard assumptions. These results demonstrate that LiSAT provides an efficient and resource-aware AI-native communication framework for semantic-adaptive waveform optimization in future 6G IoE networks.

Keywords— 6G Networks, Semantic Communications, Adaptive Waveform Optimization, Deep Reinforcement Learning, Internet of Everything (IoE)

## 1. Introduction

The vision for sixth-generation (6G) wireless networks extends far beyond enhanced mobile broadband: it encompasses a hyper-connected fabric of sensors, actuators, vehicles, robots, and ambient intelligence nodes collectively forming the Internet of Everything (IoE) [1].

In such an environment, the diversity of channel conditions—from sub-6 GHz vehicular links experiencing severe Doppler spreads to sub-THz industrial channels with rich multipath and to millimeter-wave massive IoE links constrained by strict energy budgets—renders any single, statically configured waveform fundamentally inadequate [2]. OFDM, the workhorse of 4G and 5G physical layers [3], is particularly vulnerable in doubly-dispersive channels where inter-carrier interference (ICI) grows unbounded with Doppler spread [4–6].

At the same time, semantic communications (SemCom) have emerged as a paradigm shift in which the goal of transmission is not reliable bit delivery but accurate reconstruction of task-relevant meaning [7,8]. Deep joint source-channel coding (JSCC) architectures such as DeepJSCC [9] and DeepSC [10] have demonstrated that end-to-end learned transceivers can outperform separate source-channel designs in terms of semantic fidelity measured by metrics such as BLEU score or perceptual similarity. Integrating semantic compression with waveform adaptability, however, remains an open challenge—particularly under the extreme resource constraints of edge IoE devices [2].

Deep reinforcement learning (DRL) has shown promise for online radio resource management, offering the ability to adapt to non-stationary environments without explicit system models [11]. A3C-style policy gradient methods, in particular, enable asynchronous multi-worker training and have been applied to spectrum access, power control, and beamforming [12]. Their application to multi-objective waveform selection—jointly optimizing semantic efficiency, transmit power, and latency—has not been rigorously studied in the context of physically deployable embedded systems.

This paper addresses this gap by proposing LiSAT (Lightweight Semantic-Adaptive Transceiver), an AI-native physical layer framework designed to operate within the tight memory and compute budgets of ARM Cortex-M55-class microcontrollers (256 KB SRAM, 400 MHz clock). The three principal innovations are: (i) a CSL semantic codec that achieves competitive task-oriented compression in 200 KB using INT8 quantization-aware training (QAT) [13]; (ii) a MERD channel estimation strategy that embeds a permanent AFDM pilot subframe in every transmitted frame, enabling reliable delay-Doppler profiling independent of the active data waveform; and (iii) an A3C DRL agent [12] that performs multi-objective waveform selection with sub-5 μs inference latency on commodity ARM hardware [14]. Formal convergence guarantees, memory budgeting, and extensive simulation across four heterogeneous scenarios are provided.

The original contributions of this work are as follows:

1. LiSAT end-to-end architecture: We propose and validate one of the first integrated frameworks that co-optimizes a lightweight semantic codec (CSL, INT8, ~200 KB), a delay-Doppler channel estimator (MERD), and a DRL waveform selection agent (A3C, ~50 KB) as a tightly coupled system within a 256 KB SRAM ARM Cortex-M55 microcontroller. To the best of the authors’ knowledge, this is among a novel framework to jointly integrate semantic waveform selection and deep reinforcement learning under severe embedded hardware constraints.

2. MERD delay-Doppler estimation with permanent AFDM pilot subframe: We introduce a multi-waveform channel estimation strategy in which a dedicated AFDM pilot subframe is embedded in every transmitted frame, providing consistent delay-Doppler channel state information irrespective of which data waveform is active. This design resolves the circular dependency between channel estimation and waveform selection in adaptive transceivers.

3. Formal convergence analysis with PAC-Bayes generalization bound: We provide a formal proof of convergence of the A3C agent to a local stationary ε-optimal policy (Theorem 1, Bhatnagar et al. [15]), and derive a PAC-Bayes generalization bound (Eq. 16) that quantifies policy transferability to out-of-distribution channel conditions. This theoretical foundation—rare in the semantic communications literature—distinguishes LiSAT from empirically evaluated systems such as DeepJSCC [9] and DeepSC [10].

The remainder of this paper is organized as follows. Section 2 reviews the semantic communications framework and related work. Section 3 characterizes the candidate waveforms. Section 4 details the LiSAT architecture. Section 5 provides convergence analysis. Section 6 presents performance evaluation. Section 7 discusses open problems and Section 8 concludes.

## Semantic Communication Framework

  - Principles of Semantic Communications

Classical information theory treats the meaning of transmitted symbols as irrelevant, optimizing only reliable bit transfer [16]. Semantic communication, by contrast, targets the accurate conveyance of meaning or task-relevant information to the receiver [7]. Formally, let x∈X denote a source (e.g., an image or sensor feature vector) and x∈X its reconstruction. The semantic efficiency ηs is defined as a task-specific fidelity measure—such as normalized BLEU score for text, structural similarity index (SSIM) for images, or classification accuracy for perception tasks—normalized to the unit interval. Unlike bit-error rate (BER), ηs captures perceptual and functional quality [8].

Deep JSCC architectures parameterize the encoder fϕ:X→Ck and decoder gψ:Ck→X as neural networks trained end-to-end to minimize ELx,gψfϕx+n, where n models channel noise [9]. O'Shea and Hoydis [17] demonstrated that such autoencoders learn constellation geometries adapted to channel statistics. Task-oriented multi-user extensions [10,18,19] show that semantic compression can be shared across users to exploit redundancy. Hoydis et al. [20] discuss the broader vision of an AI-native air interface for 6G in which such learned components replace hand-crafted signal processing blocks. The framework in this paper extends these ideas to the problem of adaptive waveform selection under doubly-dispersive channels.

  - Waveform Adaptability in 6G

The 6G channel landscape presents conditions that vary orders of magnitude across deployment scenarios [2]. Vehicular channels at 5.9 GHz with speeds up to 500 km/h generate Doppler shifts of order νmax≈2.75 kHz, causing OFDM ICI floors above 10-2 SER. Industrial sub-THz channels at 300 GHz exhibit rich frequency selectivity with small coherence bandwidth but modest Doppler (νmax≈80 Hz). Massive IoE channels at 28 GHz require minimal energy per frame (Ebudget=1 μJ/frame) and must serve 104 heterogeneous devices. Adapting the waveform to channel statistics is thus essential, but prior work has addressed individual scenarios in isolation or required full channel state information unavailable in embedded systems.

## Related Work and Positioning

Qin et al. [8] provide a comprehensive survey of semantic communication principles and challenges. SemCom-T [10] and DeepSC achieve high semantic fidelity for text but assume static OFDM channels. DeepJSCC [9] optimizes image transmission over AWGN and fading channels but does not adapt its waveform. Ali et al. [21]. propose a semantic-aware resource allocation framework for multi-user 6G systems. O-RAN architectures [22] provide a platform for closed-loop AI-driven radio resource management, but physical layer waveform adaptation remains outside their scope. Grassucci et al. [23]  explore generative semantic communications using diffusion models, pointing to future directions. LiSAT differentiates itself by being an integrated framework to jointly optimize semantic codec compression, delay-Doppler channel estimation with a dedicated pilot structure, and multi-waveform selection within an embedded footprint of 256 KB.

## Candidate Waveforms for Doubly-Dispersive Channels

## Channel Model

A doubly-dispersive channel in discrete time is characterized by its delay-Doppler spreading function Hτ,ν, where τ and ν denote delay and Doppler shift, respectively. The input-output relation is:

| yn=l=0L-1k=-KKhl,k xn-l ej2πkn/N+wn | (1) |
| --- | --- |

where hl,k is the channel coefficient at delay l and Doppler bin k, N is the block length, and wn~CN0,σw2 is additive white Gaussian noise. The channel is characterized by its maximum delay spread τmax and maximum Doppler spread νmax  [24].

## OTFS

Orthogonal Time Frequency Space (OTFS) modulation [25] multiplexes information symbols on a delay-Doppler grid using the inverse simplistic finite Fourier transform (ISFFT) followed by a Heisenberg transform. For a grid of M×N symbols (M delay bins, N Doppler bins), the transmitted signal is:

| sn,m=1MNk=0N-1l=0M-1Xk,l ej2πnkN-mlM | (2) |
| --- | --- |

OTFS achieves full channel diversity in the delay-Doppler domain: each symbol experiences the full time-frequency diversity of the channel, making it particularly robust to high Doppler [25]. Its main drawbacks are receiver complexity OMNlogMN and peak-to-average power ratio (PAPR) comparable to OFDM.

## OCDM

Orthogonal Chirp Division Multiplexing (OCDM)[26] uses the discrete Fresnel transform (DFnT) as its spreading basis, exploiting linear frequency-modulated (chirp) carriers. Each subcarrier sweeps the entire available bandwidth, providing inherent frequency diversity. OCDM exhibits lower PAPR than OTFS in many configurations and offers robustness to flat fading, though its orthogonality can be partially degraded by high Doppler spreads. Its complexity scales as ONlogN.

## AFDM

Affine Frequency Division Multiplexing (AFDM) [5] employs chirp-based subcarriers defined by the discrete affine Fourier transform (DAFT), parameterized by a pre-chirp c1 and post-chirp c2:

| xAFDMn=1Nm=0N-1Xm ej2πc1n2N+mnN+c2m2 | (3) |
| --- | --- |

By tuning c1 to satisfy c1=1/2Nc where Nc covers the maximum Doppler-delay product, AFDM guarantees full diversity in doubly-dispersive channels while enabling efficient delay-Doppler channel estimation through embedded pilot design [5]. Specifically, the normalized exponent c1n2/N (cf. Proposition 1 of  [5]) ensures that the Doppler resolution Δν=1/NTs is proportional to the subcarrier spacing. Its inherent parametric pilot structure makes it the natural choice for the MERD pilot subframe in LiSAT.

## Summary Comparison

Table 1 summarizes the tradeoffs among the three candidate waveforms and OFDM.

Table 1

# Waveform comparison for doubly-dispersive channels

| Waveform | Transform Basis | ICI Robustness | Receiver Complexity | PAPR | Best Scenario |
| --- | --- | --- | --- | --- | --- |
| OFDM | DFT | Low (ICI floor) | ONlogN | High | Static/low-mobility |
| OTFS | ISFFT + Heisenberg | Very High (full diversity) | OMNlogMN | Medium-High | Extreme Doppler (V2X) |
| OCDM | DFnT (chirp) | Medium-High | ONlogN | Medium | Frequency-selective (THz) |
| AFDM | DAFT (affine chirp) | High (full diversity, tunable) | ONlogN | Medium | General doubly-dispersive; pilot embedding |

## LiSAT Architecture

## System Overview

The LiSAT framework operates as follows. At the transmitter, the source data x is compressed by the CSL encoder into a latent representation z∈Ck. A dedicated MERD pilot subframe—always modulated as AFDM—is prepended to each frame, while the data payload z is modulated using the waveform selected by the A3C agent based on the previous frame's channel estimate τmax,νmax and semantic quality feedback ηs,τe2e. At the receiver, the MERD estimator extracts the delay-Doppler profile from the AFDM pilot subframe, the selected demodulator recovers z, and the CSL decoder reconstructs x. Semantic quality metrics are fed back over a low-rate control channel to the A3C agent. Figure 1 illustrates the complete block diagram.

Fig. 1. Block diagram of the LiSAT transceiver.

## Lightweight Semantic Codec (CSL)

The CSL is a convolutional autoencoder [27] designed for extreme compression under severe channel impairments. The encoder fϕ consists of four convolutional layers with kernel sizes 3,3,3,1, strides 2,2,1,1, and channel depths 32,64,128,kz, followed by a channel normalization layer that constrains the transmit power:

| z=Pt·fϕx‖fϕ(x)‖2 | (4) |
| --- | --- |

The decoder gψ mirrors the encoder with transposed convolutions. The codec is trained end-to-end using a composite loss:

| LCSL=λrec·Ltask(x,x)+λrate·‖z‖0 | (5) |
| --- | --- |

here Ltask is a task-specific fidelity loss (e.g., MSE for regression tasks, cross-entropy for classification). Following O'Shea and Hoydis [17], the channel is modeled as a stochastic layer during training, with noise variance matched to the target SNR distribution.

Quantization-Aware Training (QAT). To meet the 256 KB SRAM constraint of the ARM Cortex-M55, all weights and activations are quantized to INT8 during training using straight-through estimators for gradients [13]. The total parameter count is approximately 0.2 million, yielding a model footprint of 200 KB. INT8 QAT introduces only 1.8% SER degradation relative to the FP32 baseline—a negligible penalty for the 4× memory reduction it enables. Inference uses CMSIS-NN INT8 kernels [28] for optimized convolution on ARM Cortex-M. While transformer-based architectures [29] have shown strong performance in semantic communication, their memory footprint exceeds the 256 KB SRAM budget; hence we adopt a convolutional design inspired by residual networks [30]. The CSL architecture is inspired by the autoencoder concept of [17] and the task-oriented multi-user design of [18]. Table 2 details the memory budget for the complete LiSAT system.

Table 2

LiSAT memory budget on arm cortex-m55 (256 kb sram)

| Component | Size |
| --- | --- |
| CSL encoder/decoder INT8 weights | 200 KB |
| A3C agent INT8 weights (GRU + FC) | ~50 KB |
| MERD DFT/DAFT computation buffers | ~3 KB |
| Network activation buffers | ~2 KB |
| OS/runtime stack | ~1 KB |
| Total | ≤ 256 KB |

## Multi-Waveform Channel Estimation Strategy (MERD)

A fundamental challenge in multi-waveform adaptive transceivers is that the channel estimator must be consistent regardless of which data waveform is currently active. The MERD solves this by embedding a permanent AFDM pilot subframe at the beginning of every transmitted frame, independent of the A3C waveform selection for the data payload.

The pilot subframe occupies Np DAFT-domain symbols arranged as a cross-shaped impulse [5] in the delay-Doppler plane, surrounded by guard intervals of size τmax/Δτ and νmax/Δν. At the receiver, the AFDM pilot subframe is always demodulated first using the DAFT inverse. Building on the discrete channel model of Eq. (1), where hl,k represents the channel coefficient at delay bin l and Doppler bin k, the delay-Doppler spreading function is estimated via:

| Hl,k=Ypl,kXpl,k, l,k∈Gpilot | (6) |
| --- | --- |

where Yp and Xp are the received and transmitted pilot symbols in the DAFT domain, and Gpilot is the set of active pilot positions. The delay-Doppler profile estimates τmax and νmax are then extracted as:

| τmax=maxl·Δτ:Hl,k>δH | (7) |
| --- | --- |
| νmax=maxk·Δν:Hl,k>δH | (8) |

for a detection threshold δH. The overhead of the pilot subframe is:

| ηoh=NpNp+Nd≈8–12% | (9) |
| --- | --- |

depending on the maximum supportable delay-Doppler spread. This design guarantees that the A3C agent always receives accurate channel state even as the data waveform changes frame-to-frame, resolving the circular dependency between channel estimation and waveform selection. Crucially, the MERD operates solely on the AFDM pilot subframe; it does not require the data waveform to be AFDM.

## A3C Multi-Objective DRL Agent

The A3C agent [12,31] maintains an actor policy πθatst and a critic value function Vωst, both parameterized by unidirectional GRU networks. The reference hardware platform is the ARM Cortex-M55 CPU at 400 MHz with 256 KB SRAM [14]; the co-processor Ethos-U55 NPU is not part of the target platform because it only accelerates CNN inference and cannot execute recurrent (GRU) networks. The GRU architecture is chosen over bidirectional LSTM for two additional reasons: (a) bidirectional processing requires buffering the entire sequence before producing output, which is incompatible with causal real-time waveform selection; (b) the unidirectional GRU reduces recurrent parameter count by approximately 33% versus a bidirectional LSTM of equal hidden size. With ~50K INT8 parameters and two GRU layers of hidden size 128, the inference time on the Cortex-M55 CPU is τinfer≈2–4 μs per waveform selection step, estimated against Arm ML Evaluation Kit benchmarks and well within the 10 μs decision budget.

The actor outputs a probability distribution over the action set A=OTFS,OCDM,AFDM, and entropy regularization with coefficient βH>0 is applied throughout training to encourage exploration:

| Lactor=-E[t.logπθ(at¦st)At]-βHH[πθ] | (10) |
| --- | --- |

where At=Rt-Vωst is the advantage estimate and Hπθ is the policy entropy. The critic is trained by minimizing the mean squared Bellman error. Asynchronous workers run Nw=8 parallel environment instances, each using a different channel seed, enabling fast and stable convergence.

## MDP Formulation

The waveform selection problem is formulated as a Markov Decision Process S,A,r,P,γ:

State space: st=[τmax,t, νmax,t, SNRt, ηs,t-1, Pt,t-1, wt-1]⊤∈R6, where the subscript t-1 indicates values from the previous frame and wt-1∈0,1,2 encodes the previously selected waveform.

Action space: at∈A=OTFS,OCDM,AFDM.

Reward function: The reward is designed to jointly optimize semantic efficiency, transmit power, and end-to-end latency while penalizing constraint violations. To ensure numerical stability of policy gradient updates, all terms are explicitly normalized and the composite reward is clipped:

| rt=clip α·Δηs+β·-ΔPtPmax-γ·τe2eτmax-λ· 1constraint violated, -1, 1 | (11) |
| --- | --- |

where: Δηs=ηs,t-ηs,t-1∈-1,1 is the semantic efficiency improvement; ΔPt/Pmax∈-1,0 is the normalized power reduction; τe2e/τmax∈0,1 is the normalized end-to-end latency; and 1constraint violated∈0,1 penalizes violations of the maximum latency constraint τmax=80 μs. The weights α,β,γ,λ=0.5,0.3,0.1,0.5 are set by grid search on the V2X validation set and kept constant across all scenarios. The clip·,-1,1 operation prevents outlier rewards from destabilizing gradient estimates.

Transition model: The environment simulator models each scenario's channel using the appropriate statistical model (Section 5.1), transitions the state via MERD estimation, and returns the reward.

Discount factor: γ=0.99.

## Convergence Analysis

## Policy Gradient Setup

The A3C agent optimizes the expected discounted return:

| J(θ)=Eπθ [t=0∞γtrt] | (12) |
| --- | --- |

with stochastic gradient estimates obtained from n-step returns:

| ∇θJ≈E [t∇θlogπθ(at¦st)·At] | (13) |
| --- | --- |

Because the GRU value function approximator introduces a systematic bias, the true gradient satisfies:

| ∇θJ=∇θJ+ξMC+εapprox | (14) |
| --- | --- |

where ξMC is the Monte Carlo variance of the n-step return estimator and εapprox≠0 is the function approximation error. Consequently, convergence cannot in general be guaranteed to the global optimum of the non-convex objective Jθ.

## Theorem 1 — Convergence to a Local Stationary ε-Optimal Policy

Intuitive Summary. Before stating the formal conditions, we offer an informal preview: Theorem 1 guarantees that the A3C learning process does not diverge—that is, the policy gradient steps will eventually become negligibly small, settling at a locally optimal policy. The key ingredients are: (i) a bounded reward signal (ensured by the clip operation in Eq. (11)), (ii) a decaying step-size schedule that prevents overshooting, and (iii) sufficient richness of the GRU function approximator. The theorem does not guarantee that the algorithm finds the globally best policy, since the objective landscape is non-convex; in practice, we observe that the local optima found are within 3 dB of the oracle, suggesting they are near-globally optimal for these channel instances.

Conditions:

- (C1) The reward function rt is bounded: rt≤Rmax<∞ (satisfied by the clip operation in (11)).

- (C2) The learning rates αt satisfy t.αt=∞, t.αt2<∞ (Robbins-Monro conditions for stepsize schedule; note however that convergence to a stationary point rather than a root is proved via Bhatnagar et al. [15]).

- (C3) The policy πθ is continuously differentiable in θ and satisfies a Lipschitz condition with constant Lπ<∞.

- (C4) The MDP satisfies ergodicity: every state s∈S is reachable from any other under πθ with positive probability (cf. [32] for ergodic fading channel theory).

- (C5) The GRU function approximator has sufficient capacity to represent the optimal policy, with approximation error bounded by ‖εapprox‖2≤εa<∞.

Theorem 1. Under conditions (C1)–(C5), the A3C updates (13)–(14) with n-step returns and entropy regularization (10)  converge, as the number of episodes Nep→∞, to a local stationary ε-optimal point satisfying:

| ∇θJπθ2≤ε+Oεa | (15) |
| --- | --- |

where ε> 0 depends on the Monte Carlo variance ξMC and the learning rate schedule, and Oεa is the residual bias due to function approximation.

Proof sketch. The proof follows the framework of Bhatnagar et al. [15] for two-timescale stochastic approximation. The critic is updated on the fast timescale (ω), and the actor on the slow timescale (θ). On the fast timescale, the critic converges to the fixed-point of the projected Bellman operator within an εa-ball of the true value function (condition C5). Given this convergence, the actor gradient estimate is bounded in l2-norm by ‖∇θJ‖2≤‖∇θJ‖2+O(εa). Standard ODE analysis [15] then yields (15). Note that non-convexity of Jθ implies convergence only to a local—not global—stationary point; multiple random initializations are used in practice to mitigate this. See also Tsitsiklis and Van Roy [33] for the linear function approximation special case.

Remark on Channel Non-Stationarity. Condition (C4) requires ergodicity of the MDP, which in practice means that the channel statistics are approximately stationary over the training horizon. The Gauss-Markov model of Eq. (17) introduces time-varying statistics: the correlation parameter ρ decreases as the UAV maneuvers, effectively changing the steady-state distribution of the Markov chain over long time horizons. In the strict sense, this violates (C4). However, the non-stationarity in the simulated UAV scenario is slow relative to the A3C update timescale: the channel coherence time exceeds 103 frames, while the A3C policy is updated every n=5 frames. In this quasi-stationary regime, (C4) holds approximately, and the convergence guarantee of (15) applies in a locally time-averaged sense. For scenarios with rapid statistical jumps (e.g., indoor-to-outdoor transitions), condition (C4) may be violated; continual learning extensions are discussed in Section 7.5.

Remark. The convergence guarantees of (15) does not claim global optimality. In practice, we observe across all four scenarios that A3C converges within Nep≤2×104 episodes to policies that are within 3 dB of the genie-aided oracle (Section 6.5), suggesting the local optima found are near-globally-optimal for these problem instances.

## OOD Generalization via PAC-Bayes Analysis

To bound the policy's generalization to out-of-distribution (OOD) channel conditions—e.g., UAV channels not seen during training—we apply a PAC-Bayes analysis on the posterior policy divergence. The KL divergence bound (16) is measured empirically as follows: after training over Nep=2×104 episodes, we compute, for each state s in the validation set, the KL divergence between the learned policy softmax distribution πLiSAT·s and a uniform prior πprior=1/3,1/3,1/3 over the three waveforms. The average and 95th-percentile of this per-state divergence are 0.31 nats and 0.48 nats respectively, yielding the observed upper bound:

| DKL(πLiSAT‖πprior)≤0.5 nats | (16) |
| --- | --- |

This bound is consistent with the entropy regularization applied during A3C training (coefficient βH>0 in (10)), which prevents the policy from collapsing to a deterministic mapping and maintains policy diversity. The PAC-Bayes generalization bound implies that the expected semantic utility under unseen channel realizations deviates from the training-set utility by at most ODKL/Nep, which is negligible for Nep≥104. The empirical evidence in Section 6.5 (UAV fine-tuning requiring only 350 episodes) is consistent with this analysis.

## Performance Evaluation

## Simulation Setup

All simulations were implemented in Python using PyTorch 2.0 for neural network training and a custom physical-layer simulator for waveform generation and channel modeling. The CMSIS-NN INT8 inference library  [28] was benchmarked on an ARM Cortex-M55 development platform operating at 400 MHz with 256 KB of SRAM. To promote transparency and reproducibility, all simulation scripts, trained models, waveform generation modules, and evaluation routines used in this work are publicly available in the LiSAT repository LiSAT GitHub repository,  enabling independent reproduction of the reported results. The repository enables full reproduction of the experimental results and performance analyses reported in this paper. Table 3 summarizes the simulation parameters and channel configurations adopted for each evaluated scenario.

Table 3

Simulation Parameters and Channel Profiles

| Parameter | V2X | THz Industrial | Massive IoE | UAV Non-Stationary |
| --- | --- | --- | --- | --- |
| Carrier frequency | 5.9 GHz | 300 GHz | 28 GHz | 3.5 GHz |
| Channel model | ITU-R VehicularB | 3GPP TDL-C (adapted) | 3GPP TDL-A | Gauss-Markov non-stat. |
| # multipath components | 6 | 12 | 8 | Variable |
| τrms | 650 ns | 50 ns | 100 ns | Variable |
| Max. speed / νmax | 500 km/h / 2.75 kHz | Static–3 m/s / 80 Hz | Variable | Variable |
| Bandwidth | 10 MHz | 2 GHz | 100 MHz | 20 MHz |
| MERD pilot overhead ηoh | 10% | 8% | 8% | 12% |
| Pmax | 30 dBm | 20 dBm | 10 dBm | 27 dBm |
| τmax (latency constraint) | 80 μs | 100 μs | 200 μs | 80 μs |
| Ebudget | — | — | 1 μJ/frame | — |
| A3C training episodes | 16,000 | 12,000 | 8,500 | 18,000 |
| Random seeds | 5 | 5 | 5 | 5 |

The THz industrial and massive IoE scenarios use 3GPP TDL-C and TDL-A channel models, respectively [34]. Evaluation parameters follow the guidelines of [35] for IMT-2020 radio interface technologies.

The CSL codec is trained for 100 epochs with the Adam optimizer [36] (learning rate 10-3, batch size 128) on a dataset of 50,000 source samples per scenario. The A3C agent uses 8 asynchronous workers, n-step returns with n=5, entropy coefficient βH=0.01, and αactor,αcritic=10-4,5×10-4. The two-timescale formulation of Theorem 1 (Bhatnagar et al. [15]) requires αcritic>αactor to ensure the critic converges on the fast timescale. These values reflect this requirement and are the authoritative hyperparameters for all reported results.

## V2X High-Mobility Results

The V2X scenario—carrier 5.9 GHz, vehicle speed 500 km/h, ITU-R VehicularB channel with 6 paths, νmax=2.75 kHz—is the most demanding in terms of Doppler-induced ICI. OFDM suffers a well-known ICI floor that prevents SER from falling below approximately 10-1.5 at any SNR.

Figure 2 shows BER/SER versus SNR curves from −5 dB to 30 dB for all methods. LiSAT achieves SER = 0.077 at SNR = 15 dB, compared to SER = 0.100 for OFDM—a 23% reduction. All SER/BER curves are generated with NMC=1000 Monte Carlo iterations per SNR point; the resulting 95% confidence interval at SER ≈0.077 is approximately ±0.008, confirming statistical significance of the reported gains. The AFDM fixed waveform closely approaches LiSAT performance in this scenario, and indeed LiSAT selects AFDM 78% of the time, OTFS 15%, and OCDM 7%. The CSI-threshold heuristic, which switches to AFDM when νmax>1 kHz, achieves intermediate performance but lacks the semantic compression benefits of CSL. The genie-aided oracle (omniscient waveform selection with infinite-precision channel knowledge) lies within 1.5 dB of LiSAT at high SNR, confirming the near-optimality of the learned policy. OFDM shows an ICI floor beyond 20 dB SNR that LiSAT avoids entirely.

Fig. 2. BER/SER versus SNR (−5 dB to 30 dB), V2X scenario (5.9 GHz, 500 km/h, ITU-R Vehicular B).

In terms of transmit power, LiSAT achieves Pt=18.6 dBm versus 23.2 dBm for OFDM at equivalent SER—a 31% reduction enabled by the CSL's semantic compression reducing the payload size. The end-to-end latency constraint τe2e<80 μs is satisfied in 96.3% of frames, with the rare violations occurring during abrupt Doppler transients between handoffs.

## Industrial THz Results

The THz industrial scenario (300 GHz, 3GPP TDL-C adapted, τrms=50 ns, νmax=80 Hz) is dominated by frequency selectivity rather than Doppler spread. OCDM's full-band chirp diversity is well matched to this channel, and LiSAT selects OCDM 61% of the time, AFDM 28%, and OTFS 11%.

The key metric in this scenario is semantic efficiency ηs. LiSAT achieves ηs=0.87 versus ηs=0.71 for OFDM—a +22.5% gain that reflects both the CSL's semantic compression and the improved channel capacity enabled by OCDM's chirp diversity. The end-to-end latency is τe2e=65 μs, well within the 100 μs constraint. The MERD pilot overhead of 8% is acceptable given the large bandwidth (2 GHz), representing ≈160 MHz of pilot resources—sufficient to resolve the dense multipath structure.

## Massive IoE Results

The massive IoE scenario (28 GHz, 3GPP TDL-A, 104 heterogeneous devices, Ebudget=1 μJ/frame) evaluates aggregate performance across a large device population. The energy budget of 1 μJ/frame and the need for ultrareliable communication in IoE are consistent with short-packet transmission theory [37,38]. We define the aggregate semantic efficiency ηsagg as the 10th-percentile semantic utility across all active devices—a metric that captures the worst-case tail performance relevant for reliability-constrained IoE applications.

LiSAT achieves ηsagg>0.75 at the 10th percentile, with individual device transmit power Pt=1.38 mW versus 2.0 mW for OFDM (−31% energy reduction). The A3C agent converges in 8,500 episodes—the fastest of all four scenarios—attributed to the relatively benign channel statistics of TDL-A and the tight clustering of device states in the MDP state space.

Multiple Access Interference (MAI) Limitation. The LiSAT framework assumes that devices operate independently, ignoring inter-device interference. In practice, a dense deployment of 104 devices in a shared spectrum will exhibit significant MAI. Modeling the interference field as a spatial Poisson process with density λd, the SINR at a representative receiver degrades due to aggregate interference power Iagg=πλdβ0/αpath-2 (where β0 is the reference path gain and αpath>2 the path loss exponent). Our Monte Carlo extrapolation—using the SNR-to-ηs transfer curve measured in the isolated-device scenario—suggests that this SINR reduction translates to a decrease of approximately 5–10 percentage points in ηsagg for typical dense IoE densities (λd∈103,104 devices/ km2). This estimate should be validated by explicit MAI simulation, which is left as future work. Future work should also explore multi-agent DRL formulations where devices cooperatively manage interference and waveform selection.

## Non-Stationary UAV Results

The UAV scenario (3.5 GHz, Gauss-Markov non-stationary channel model with variable Doppler and continuously changing path geometry) tests LiSAT's adaptability to channels whose statistics evolve over time. The Gauss-Markov model introduces time-correlated channel variations:

| ht+1=ρht+1-ρ2 ξt, ξt~CN0,Σh | (17) |
| --- | --- |

where ρ∈0,1 controls temporal correlation and decreases as the UAV maneuvers.

Figure 3 shows that A3C agent converges in 18,000 episodes—the slowest scenario due to the non-stationarity. The BER gap to the genie-aided oracle is less than 3 dB at high SNR, demonstrating that the learned policy tracks channel variations effectively. The latency constraint τe2e<80 μs is met in 94.8% of frames.

Transfer Learning. Fine-tuning the V2X-trained LiSAT policy on UAV channel traces requires only 350 episodes to reach within 0.5 dB of the fully-trained UAV policy. This rapid adaptation is consistent with the PAC-Bayes bound (16): the small KL divergence between V2X and UAV policies implies that the learned feature representations transfer efficiently. The dominant waveform selection in this scenario is AFDM (81%), consistent with the high-mobility nature of UAV channels.

Fig. 3. A3C convergence curves (average cumulative reward vs. training episodes, 0 to 2×104) for all four scenarios.

## Comparative Summary and Waveform Selection

Table 4 provides a comprehensive performance comparison of LiSAT against semantic baselines, fixed waveforms, and the CSI-threshold heuristic.

Table 4

Performance comparison across methods (v2x scenario unless noted)

| Method | SER @ 15 dB | ηs | Pt (dBm) | τe2e (μs) | U (global utility) |
| --- | --- | --- | --- | --- | --- |
| DeepJSCC [6] (OFDM channel) | 0.095 | 0.73 | 23.2 | 78 | 0.511 |
| DeepSC / SemCom-T [7] (OFDM) | 0.091 | 0.76 | 23.2 | 82 | 0.528 |
| CSI-threshold heuristic | 0.083 | 0.79 | 21.4 | 75 | 0.591 |
| OFDM (reference) | 0.100 | 0.71 | 23.2 | 76 | 0.523 |
| OTFS (fixed) | 0.082 | 0.80 | 22.1 | 77 | 0.603 |
| OCDM (fixed) | 0.088 | 0.77 | 22.8 | 74 | 0.571 |
| AFDM (fixed) | 0.079 | 0.82 | 20.3 | 72 | 0.638 |
| LiSAT (proposed) | 0.077 | 0.87 | 18.6 | <80 (96.3%) | 0.742 |
| Genie-aided oracle | 0.071 | 0.90 | 17.8 | 70 | 0.781 |

Global utility U is a weighted composite of ηs, -Pt, -SER, and τe2e-1 normalized to [0,1]. DeepJSCC and DeepSC results assume OFDM channel without waveform adaptation; their performance degrades further in high-Doppler scenarios. τe2e for DeepSC exceeds the 80 μs constraint due to higher decoder complexity.

LiSAT achieves a global utility of U=0.742 versus U=0.523 for OFDM—a +41.9% improvement. It outperforms the semantic baselines DeepJSCC and DeepSC primarily because those systems do not adapt their waveform to doubly-dispersive channels. The CSI-threshold heuristic, while competitive in V2X, lacks multi-objective optimization and semantic compression, yielding U=0.591.

Baseline Fairness Qualification. The results in Table 4 evaluate DeepJSCC [9] and DeepSC [10] under OFDM modulation only, without the benefit of waveform adaptation. This is intentional: DeepJSCC and DeepSC were designed and trained specifically for AWGN or flat-fading OFDM channels, and adapting them to doubly-dispersive waveforms would require retraining their end-to-end architectures—a fundamentally different system than what was published. To estimate an upper bound on what waveform-adapted baselines might achieve, we conducted supplementary experiments in which the DeepJSCC channel encoder/decoder is retained but the OFDM modulator is replaced by a fixed OTFS modulator (the strongest fixed waveform in the V2X scenario). This hybrid DeepJSCC+OTFS achieves SER = 0.082 and ηs=0.79, corresponding to U=0.605. LiSAT still outperforms this hybrid (U=0.742 vs. 0.605), demonstrating that its advantage stems from the joint semantic-adaptive co-optimization of the CSL, MERD, and A3C modules—not merely from waveform selection alone. Figure 4 shows the waveform selection frequency distribution across scenarios and SNR regimes.

Fig. 4. Stacked bar chart of waveform selection frequency

## Ablation Study

Table 5 quantifies the contribution of each LiSAT component through systematic ablation.

Table 5

ablation study (v2x scenario, snr = 15 db)

| Configuration | SER | ηs | Pt (dBm) | Notes |
| --- | --- | --- | --- | --- |
| LiSAT Full | 0.077 | 0.87 | 18.6 | All modules active |
| LiSAT w/o MERD | 0.094 | 0.81 | 20.1 | Coarse CSI from data preamble only |
| LiSAT w/o CSL (bit TX) | 0.079 | 0.71 | 22.8 | No semantic compression; raw bits |
| LiSAT w/o DRL (CSI-threshold) | 0.083 | 0.79 | 21.4 | Rule-based waveform selection |
| CSL-FP32 (no INT8 QAT) | 0.076 | 0.88 | 18.5 | Full precision, 4× memory (800 KB) |

The ablation results reveal the individual contribution of each component. Removing MERD (row 2) degrades SER from 0.077 to 0.094—an increase of 22%—because the A3C agent receives only coarse channel state from the data preamble, leading to suboptimal waveform selections during rapidly changing channel conditions. Removing CSL (row 3) maintains similar SER (bit transmission is reliable) but reduces ηs from 0.87 to 0.71, confirming that the semantic gain is entirely attributed to the CSL codec. Replacing DRL with a CSI-threshold heuristic (row 4) increases SER by 8% and reduces ηs by 9%, demonstrating the value of multi-objective learned optimization over rule-based switching. The CSL-FP32 baseline (row 5) shows marginally better performance (ΔSER=1.8%) at the cost of 4× memory (800 KB)—exceeding the 256 KB constraint and confirming that QAT is essential for embedded deployment.

## Discussion and Open Problems

## Integration with O-RAN and 6G Control Planes

The LiSAT framework is designed as a device-side AI-native PHY module, but its deployment in 6G networks will require tight integration with the radio access network (RAN) control plane. O-RAN architectures [22] provide standardized interfaces (E2, O1, A1) between RAN Intelligent Controllers (RICs) and radio units, enabling closed-loop AI-driven management. LiSAT's A3C agent could be enhanced by receiving policy guidance from a near-real-time RIC that has network-wide view of channel conditions and interference, while retaining local inference capability for sub-millisecond waveform selection. The 2–4 μs inference latency of the Cortex-M55 GRU is compatible with the 1 ms subframe timing of 5G NR and the anticipated ≤ 0.1 ms subframe of 6G.

## Multi-Agent Interference Management

As noted in Section 6.4, the current LiSAT formulation assumes independent device operation, ignoring MAI. Extending to a multi-agent DRL (MADRL) formulation where K LiSAT devices cooperatively manage waveform selection and power allocation is a natural extension. Mean-field game theory [39] provides a tractable framework for large-K MADRL and could model the Poisson interference field more accurately. Federated learning [40] with differential privacy constraints could enable policy sharing across devices without exposing raw channel data—relevant for privacy-sensitive IoE applications.

## Generative Semantic Communications

Grassucci et al. [23] have recently demonstrated that diffusion-model-based semantic codecs can generate perceptually realistic reconstructions even at extreme compression ratios, far beyond what discriminative autoencoders achieve. Integrating a lightweight diffusion-based CSL into LiSAT—leveraging score-matching distillation to reduce inference steps to 1–4 could substantially increase semantic efficiency, particularly for visual data. The challenge is that diffusion models are compute-intensive; architectural co-design with the Cortex-M55 instruction set will be necessary.

## Beyond Three Waveforms

The current action space of LiSAT includes three candidate waveforms. 6G standardization may introduce additional waveforms, such as Reconfigurable Intelligent Surface (RIS)-assisted spatial multiplexing or hybrid OTFS-OFDM [6]. Extending the A3C action space to A>3 is straightforward from an MDP perspective but increases the convergence time approximately as OA1.5 empirically. Hierarchical DRL—with a high-level agent selecting waveform families and a low-level agent tuning parameter—may mitigate this.

## Channel Non-Stationarity and Lifelong Learning

The UAV scenario in Section 6.5 represents a mild form of non-stationarity. More severe non-stationarity, such as channel statistics that shift discontinuously (e.g., indoor-to-outdoor transitions), may violate condition (C4) of Theorem 1 (ergodicity). Continual or lifelong learning approaches—such as elastic weight consolidation (EWC) or progressive neural networks—could maintain performance across such transitions. The 350-episode fine-tuning time observed for V2X→UAV transfer suggests that LiSAT's representations are already somewhat general; formal continual learning integration remains for future work.

## Hardware Validation

All latency and memory figures in this paper are derived from cycle-accurate simulation on the ARM Cortex-M55 Ecosystem Development Kit and from CMSIS-NN benchmark runs on a Raspberry Pi 4 emulating Cortex-M55 instruction timing. Full chip-level validation on a silicon Cortex-M55 device is required before deployment claims can be made. In particular, the 2–4 μs inference figure assumes that the GRU weights fit in the L1 data cache (16 KB); cache misses during weight loading could increase this to 6–8 μs in worst-case scenarios.

## Conclusion

This paper has presented LiSAT (Lightweight Semantic-Adaptive Transceiver), a novel AI-native physical layer framework for 6G IoE systems that makes three original contributions. First, LiSAT introduces the first co-optimized architecture combining a Lightweight Semantic Codec (CSL, INT8, ~200 KB), a delay-Doppler channel estimator (MERD), and an A3C deep reinforcement learning agent (~50 KB) within a 256 KB SRAM footprint on an ARM Cortex-M55 processor—demonstrating that semantic-adaptive waveform design is practically realizable on commodity embedded hardware. Second, the MERD strategy—embedding a permanent AFDM pilot subframe in every transmitted frame—provides consistent delay-Doppler channel state information independently of the selected data waveform, enabling reliable multi-waveform adaptation without the circular estimation dependency that afflicts simpler designs. Third, formal convergence analysis (Theorem 1, Bhatnagar et al. [15]) proves that the A3C agent converges to a local stationary ε-optimal policy, with function approximation error bounded by Oεa; a PAC-Bayes generalization bound (Eq. 16) further quantifies the policy's transferability to unseen channel conditions, with the small 350-episode fine-tuning time for V2X→UAV transfer validating this theoretical bound empirically.

Simulation across four heterogeneous 6G scenarios demonstrates substantial performance gains: 23% SER reduction in V2X (500 km/h), 22.5% semantic efficiency gain in THz industrial links, ηsagg>0.75 at the 10th percentile for massive IoE with 10,000 devices, and BER within 3 dB of the oracle in non-stationary UAV channels. The global utility improvement of 41.9% over OFDM (U=0.742 vs. 0.523) and superior performance over semantic baselines DeepJSCC and DeepSC confirm the value of joint waveform adaptability and semantic compression. INT8 quantization-aware training introduces only 1.8% SER degradation while enabling 4× memory reduction essential for embedded deployment.

Open problems include multi-agent interference management, diffusion-based semantic codecs, continual learning for severe non-stationarity, and full silicon validation. LiSAT establishes a concrete and practically realizable baseline for AI-native physical layer design in 6G IoE systems.

Credit authorship contribution statement

Evelio Astaiza Hoyos: Conceptualization, Methodology, Software, Data curation, Formal analysis, Validation, Visualization, Writing – original draft. Héctor Fabio Bermúdez-Orozco: Conceptualization, Supervision, Methodology, Formal analysis, Validation, Writing – review & editing, Project administration. Juan Esteban Astaiza Fuenmayor: Validation, Investigation, Formal analysis, Writing – review & editing.

Declaration of competing interest

The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

Data availability

The authors have made all materials necessary to reproduce the results reported in this study publicly available in the LiSAT GitHub repository (https://github.com/eastaizah/LISAT). The repository includes the complete simulation framework, channel model implementations, waveform modulation and demodulation modules, LiSAT semantic communication and deep reinforcement learning components, configuration files, and evaluation scripts used to generate the results presented in this paper. In particular, the repository provides dedicated scripts for reproducing the BER/SER performance analysis, A3C convergence evaluation, waveform selection experiments, and the corresponding figures reported in the manuscript. Numerical results and generated figures can be automatically reproduced using the provided source code and configuration files. These resources enable independent verification and full reproducibility of the experimental results and performance analyses presented in this work. Additional information is available from the corresponding author upon reasonable request.

Declaration of Generative AI and AI-Assisted Technologies in the Writing Process

During the preparation of this manuscript, the authors used ChatGPT to improve language clarity, grammar, and readability. The authors reviewed and edited all generated content and take full responsibility for the final content of the publication.

Funding

This research received no external funding.

## References

[1]	ITU-R, IMT Towards 2030 and Beyond, International Telecommunication Union Radiocommunication Sector (ITU-R), 2023. https://www.itu.int/rec/R-REC-M.2160.

[2]	M. Giordani, M. Polese, M. Mezzavilla, S. Rangan, M. Zorzi, Toward 6G Networks: Use Cases and Technologies, IEEE Communications Magazine 58 (2020) 55–61. https://doi.org/10.1109/MCOM.001.1900411.

[3]	F. Boccardi, R.W. Heath, A. Lozano, T.L. Marzetta, P. Popovski, Five disruptive technology directions for 5G, IEEE Communications Magazine 52 (2014) 74–80. https://doi.org/10.1109/MCOM.2014.6736746.

[4]	P. Bello, Characterization of Randomly Time-Variant Linear Channels, IEEE Transactions on Communications Systems 11 (1963) 360–393. https://doi.org/10.1109/TCOM.1963.1088793.

[5]	A. Bemani, N. Ksairi, M. Kountouris, AFDM: A Full Diversity Next Generation Waveform for High Mobility Communications, (2021). https://arxiv.org/abs/2104.11331.

[6]	P. Raviteja, K.T. Phan, Y. Hong, E. Viterbo, Interference Cancellation and Iterative Detection for Orthogonal Time Frequency Space Modulation, IEEE Transactions on Wireless Communications 17 (2018) 6501–6515. https://doi.org/10.1109/TWC.2018.2860011.

[7]	C.E. Shannon, A Mathematical Theory of Communication, Bell System Technical Journal 27 (1948) 379–423. https://doi.org/10.1002/j.1538-7305.1948.tb01338.x.

[8]	Z. Qin, X. Tao, J. Lu, W. Tong, G.Y. Li, Semantic Communications: Principles and Challenges, (2022). https://arxiv.org/abs/2201.01389.

[9]	E. Bourtsoulatze, D.B. Kurka, D. Gündüz, Deep Joint Source-channel Coding for Wireless Image Transmission, in: ICASSP 2019 - 2019 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 2019: pp. 4774–4778. https://doi.org/10.1109/ICASSP.2019.8683463.

[10]	H. Xie, Z. Qin, G.Y. Li, B.-H. Juang, Deep Learning Enabled Semantic Communication Systems, IEEE Transactions on Signal Processing 69 (2021) 2663–2675. https://doi.org/10.1109/TSP.2021.3071210.

[11]	N.C. Luong, D.T. Hoang, S. Gong, D. Niyato, P. Wang, Y.-C. Liang, D.I. Kim, Applications of Deep Reinforcement Learning in Communications and Networking: A Survey, IEEE Communications Surveys & Tutorials 21 (2019) 3133–3174. https://doi.org/10.1109/COMST.2019.2916583.

[12]	V. Mnih, A.P. Badia, M. Mirza, A. Graves, T. Harley, T.P. Lillicrap, D. Silver, K. Kavukcuoglu, Asynchronous methods for deep reinforcement learning, in: Proceedings of the 33rd International Conference on International Conference on Machine Learning - Volume 48, JMLR.org, 2016: pp. 1928–1937.

[13]	B. Jacob, S. Kligys, B. Chen, M. Zhu, M. Tang, A. Howard, H. Adam, D. Kalenichenko, Quantization and Training of Neural Networks for Efficient Integer-Arithmetic-Only Inference, in: 2018 IEEE/CVF Conference on Computer Vision and Pattern Recognition, 2018: pp. 2704–2713. https://doi.org/10.1109/CVPR.2018.00286.

[14]	ARM Ltd., Cortex-M55 Technical Reference Manual, Revision r1p2, 2022. https://developer.arm.com/documentation/101051/latest/.

[15]	S. Bhatnagar, H.L. Prasad, L.A. Prashanth, Stochastic Recursive Algorithms for Optimization: Simultaneous Perturbation Methods, Springer London, London, 2013. https://doi.org/10.1007/978-1-4471-4285-0.

[16]	T.M. Cover, J.A. Thomas, Elements of Information Theory, 2nd ed., Wiley, Hoboken, NJ, 2006.

[17]	T. O’Shea, J. Hoydis, An Introduction to Deep Learning for the Physical Layer, IEEE Transactions on Cognitive Communications and Networking 3 (2017) 563–575. https://doi.org/10.1109/TCCN.2017.2758370.

[18]	H. Xie, Z. Qin, G.Y. Li, Task-Oriented Multi-User Semantic Communications for VQA, IEEE Wireless Commun. Lett. 11 (2022) 553–557. https://doi.org/10.1109/LWC.2021.3136045.

[19]	H. Xie, Z. Qin, X. Tao, K.B. Letaief, Task-Oriented Multi-User Semantic Communications, IEEE J. Select. Areas Commun. 40 (2022) 2584–2597. https://doi.org/10.1109/JSAC.2022.3191326.

[20]	J. Hoydis, F.A. Aoudia, A. Valcarce, H. Viswanathan, Toward a 6G AI-Native Air Interface, IEEE Communications Magazine 59 (2021) 76–81. https://doi.org/10.1109/MCOM.001.2001187.

[21]	A. Ali, M.U. Zahid, M.H. Shah, Semantic-Aware Resource Allocation for 6G ISAC: A DDPG-Driven Approach, (2026). https://doi.org/10.20944/preprints202601.0599.v1.

[22]	M. Polese, L. Bonati, S. D’Oro, S. Basagni, T. Melodia, Understanding O-RAN: Architecture, Interfaces, Algorithms, Security, and Research Challenges, IEEE Communications Surveys & Tutorials 25 (2023) 1376–1411. https://doi.org/10.1109/COMST.2023.3239220.

[23]	E. Grassucci, S. Barbarossa, D. Comminiello, Generative Semantic Communication: Diffusion Models Beyond Bit Recovery, (2026). https://arxiv.org/abs/2306.04321.

[24]	A. Goldsmith, Wireless Communications, Cambridge University Press, Cambridge, 2005. https://doi.org/10.1017/CBO9780511841224.

[25]	R. Hadani, S. Rakib, M. Tsatsanis, A. Monk, A.J. Goldsmith, A.F. Molisch, R. Calderbank, Orthogonal Time Frequency Space Modulation, in: 2017 IEEE Wireless Communications and Networking Conference (WCNC), 2017: pp. 1–6. https://doi.org/10.1109/WCNC.2017.7925924.

[26]	X. Ouyang, J. Zhao, Orthogonal Chirp Division Multiplexing, IEEE Transactions on Communications 64 (2016) 3946–3957. https://doi.org/10.1109/TCOMM.2016.2594792.

[27]	Y. LeCun, Y. Bengio, G. Hinton, Deep learning, Nature 521 (2015) 436–444. https://doi.org/10.1038/nature14539.

[28]	L. Lai, N. Suda, V. Chandra, CMSIS-NN: Efficient Neural Network Kernels for Arm Cortex-M CPUs, (2018). https://arxiv.org/abs/1801.06601.

[29]	A. Vaswani, N. Shazeer, N. Parmar, J. Uszkoreit, L. Jones, A.N. Gomez, Ł. Kaiser, I. Polosukhin, Attention is all you need, in: Proceedings of the 31st International Conference on Neural Information Processing Systems, Curran Associates Inc., Red Hook, NY, USA, 2017: pp. 6000–6010.

[30]	K. He, X. Zhang, S. Ren, J. Sun, Deep Residual Learning for Image Recognition, in: 2016 IEEE Conference on Computer Vision and Pattern Recognition (CVPR), 2016: pp. 770–778. https://doi.org/10.1109/CVPR.2016.90.

[31]	R.S. Sutton, A.G. Barto, Reinforcement Learning: An Introduction, 2nd ed., MIT Press, Cambridge, MA, 2018. https://web.stanford.edu/class/psych209/Readings/SuttonBartoIPRLBook2ndEd.pdf.

[32]	E. Biglieri, J. Proakis, S. Shamai, Fading channels: information-theoretic and communications aspects, IEEE Transactions on Information Theory 44 (1998) 2619–2692. https://doi.org/10.1109/18.720551.

[33]	J.N. Tsitsiklis, B. Van Roy, An analysis of temporal-difference learning with function approximation, IEEE Transactions on Automatic Control 42 (1997) 674–690. https://doi.org/10.1109/9.580874.

[34]	3rd Generation Partnership Project (3GPP), Study on Channel Model for Frequencies from 0.5 to 100 GHz, 3GPP, 2022. https://www.3gpp.org/ftp/Specs/archive/38_series/38.901/38901-h00.zip.

[35]	ITU-R, Guidelines for Evaluation of Radio Interface Technologies for IMT-2020, International Telecommunication Union Radiocommunication Sector (ITU-R), 2017. https://www.itu.int/pub/R-REP-M.2412-2017.

[36]	D.P. Kingma, L.J. Ba, Adam: A Method for Stochastic Optimization, in: Int. Conf. Learning Representations (ICLR), Ithaca, NY:ArXiv, 2015. https://dare.uva.nl/search?identifier=a20791d3-1aff-464a-8544-268383c33a75 (accessed April 18, 2026).

[37]	P. Zhu, J. Li, D. Wang, X. You, Short-Packet Transmission in Multiple Antenna Systems: Capacity Bounds and Code Design Challenges, IEEE Wireless Communications 27 (2020) 68–75. https://doi.org/10.1109/MWC.001.1900516.

[38]	G. Durisi, T. Koch, P. Popovski, Toward Massive, Ultrareliable, and Low-Latency Wireless Communication With Short Packets, Proceedings of the IEEE 104 (2016) 1711–1726. https://doi.org/10.1109/JPROC.2016.2537298.

[39]	M. Huang, R.P. Malhamé, P.E. Caines, Large population stochastic dynamic games: closed-loop McKean-Vlasov systems and the Nash certainty equivalence principle, Cis 6 (2006) 221–252.

[40]	B. McMahan, E. Moore, D. Ramage, S. Hampson, B.A. y Arcas, Communication-Efficient Learning of Deep Networks from Decentralized Data, in: Artificial Intelligence and Statistics, PMLR, 2017: pp. 1273–1282. https://proceedings.mlr.press/v54/mcmahan17a.html (accessed May 11, 2026).